# ELECTRONIC BALLAST SPECIFICATION

- General Description

This high-intensity-discharge metal-halide lamp ballast, is designed using constant power and current regulation technologies. The warm up current and power output is well controlled to meet the specification of modern HID lamps. Both are adjustable to adapt variable type of lamps. Proper venting is required to keep the normal operation, Essential protections are incorporated to ensure safety of operation and aging lamp detection.

| TPCI P/N : TBHID1TI-01 | 150W |
| --- | --- |
| TPCI P/N : TBHID1TI-02 | 120W |
| TPCI P/N : TBHID1TI-03 | 100W |

- Applicable Standards

  - Safety Standards

UL 1950 (Pending)

IEC 950 (Pending)

CSA 950 (Pending)

EN 60950 (Pending)

2.2 Radiation

FCC Class B (Pending)

EN 55022 Class B (Pending)

BSMI (Pending)

3.Electrical

3.1 Input Section

| Input Voltage | 370 ± 30Vdc |
| --- | --- |
| Input Current | 150Watts lamp@350Vdc 0.48A typ. |
|  | 120Watts lamp@350Vdc 0.38A typ. |
|  | 100Watts lamp@320Vdc 0.32A typ. |
| Electric Conversion efficiency | 88% typ.. |

3.2 Output Section

| Ignition | Voltage | 17± 3KV |
| --- | --- | --- |
|  | Strike rate | 20 pulses per second, min. |
|  | Striking time | 3 sec, min. |
| Warm Up Current |  | 3.2A to 4.2A adjustable |
| Lamp Voltage |  | 40Vpp ~ 240Vpp (square wave ) or 20Vdc ~ 120Vdc (Amplitude) |
| Inverter Frequency |  | 200Hz ± 7Hz |
| Output Power |  | 100W ~ 150W adjustable |
| Output Power Deviation |  | ± 2W |

3.3 Signal Section

| Turn-On Signal pin | Turn-On | Von | 2.4V~5V |
| --- | --- | --- | --- |
|  |  | Iin,on | 1.4mA~5mA |
|  | Turn-Off | Voff | <0.5V |
|  |  | Iin,off | -- |
| SUCCESS Signal pin | Starting ok | Vce | Vce,sat<0.3 |
|  |  | Ic | 10mA max. |
|  | Fault occurs | Vce | Open, Vceo = 30Vmax |
|  |  | Ic | -- |

The SUCCESS signal will be presented correctly after 3 seconds while TURN-ON signal activated.

3.3 Applicable Lamp

High Intensity Discharge Metal Halide Lamp

Suggested operation lamp voltage : 50V~90V, 75V typical

Wattage : 100W ~ 150W

3.4 Interface

3.4.1 connector pin assignment

DC voltage input connector (CN900)

Type : JST B2P3-VH-B

| PIN1 | Vin+( 350Vdc~400Vdc) |
| --- | --- |
| PIN2 | Omit |
| PIN3 | Vin- (GND) |

Control I/O connector (CN901)

Type : JST B4B-EH

| PIN1 | SUCESS signal ( SIG ) |
| --- | --- |
| PIN2 | Signal GND ( SGND ) |
| PIN3 | TURN-ON signal ( ON ) |

Note : Power GND and Signal GND are isolated

3.4.2 Control Signal Circuit Diagram

3.4.3 Control Sequence

SUCCESS signal is pull high

3.5 Protection

A. Over Temperature Protection : activated while fan venting is fail or missing.

B. Over Voltage Protection : activated while lamp voltage is over 120Volts

( The aging lamp voltage will increase as respect to the time of using, this protection voltage(120V) is assumed the lamp is running to the failure point and the lamp has to be replaced)

C. Stricking time Protection : activated after the ignitor striking for 3 seconds or over

All above protection can be reset by toggle the turn-on signal off and on.

- Mechanical

4.1 Outline

4.2 Weight

4.3 Package

5. Environment

5.1 Operation Temperature

5.2 Storage Temperature