PERFORMANCE – ALL APPLIANCES

28 Test Voltages

28.1 Unless otherwise specified,products shall be tested at the appropriate potential indicated in Table.

28.2 for each test as detailed in the describing the test.

Table 28.1

Test Voltages

| Rated voltage | Input test voltage | Temperature test voltage | Blade ignition test voltage |
| --- | --- | --- | --- |
| 110―120 191―208 210―240 254―277 440―480 550―600 | Rated Rated Rated Rated Rated Rated | 120 208 240 277 480 600 | 120 208 240 277 480 600 |
| For nameplate rating other than one of those specified in the table,the motor is to be tested at 100―105 percent of the rated voltage.However,consideration is to be given to the voltage of the intended source of supply for a motor. |  |  |  |

29 Leakage Current Test

29.1 A cord-connected product rated for a nominal 240-volt or less supply shall be tested in accordance with 29.3-29.6.Leakage current shall not exceed:

a) 0.5 milliampere for an ungrounded 2-wire product;

b) 0.5 milliampere for a grounded,3-wire,portable product;and

c) 0.75 milliampere for a grounded,3-wire,product:

1) Employing a standard attachment plug rated 20 amperes or less;and

2) Intended to be fastened in place or located in a dedicated space.

29.2 Leakage current refers to all currents,including capacitively coupled currents,that may be conveyed between exposed conductive surfaces of an appliance and ground ot other exposed conductive surfaces of an appliance.

29.3 All exposed conductive surfaces are to be tested for leakage currents.The leakage currents from these surfaces are to be measured to the grounded supply conductor individually as well as collectively where simultaneously accessible.Parts are considered to be exposed surfaces unless guarded by an enclosure considered acceptable for reducing the risk of electric shock as defined in 9.1-9.5.Surfaces are considered to be simultaneously accessible if they can be readily contacted by one or both hands of a person at the same time.These measurements do not apply to terminals operating at voltages that are not considered to involve a risk of electric shock.If all accessible surfaces are bonded together and connected to the grounding conductor of the power-supply cord,the leakage current can be measured between the grounding conductor and the grounded supply conductor.

29.4 If a conductive surface other than metal is used for the enclosure or part of the enclosure,the leakage current is to be measured using metal foil having an area of 10 by 20 centimeters (3.9 by 7.9 inches) in contact with the surface.If the surface has an area of less than 10 by 20 centimeters (3.9 by 7.9 inches),the metal foil is to be the same size as the surface.The metal foil is not to remain in place long enough to affect the temperature of the appliance.

29.5 The measurement circuit for leakage current is to be as illustrated in Figure 29.1.The measurement instrument is defined in (a) – (d).The meter that is actually used for a measurement need not only indicate the same numerical value for a particular measurement as would the defined instrument.The meter used need not have all the attributes of the defined instrument:

a)The meter is to have an input impedance of 1500 ohms resistive shunted by a capacitance of 0.15 microfarad.

b)The meter is to indicate 1.11 times the average of the full-wave rectified composite waveform of the voltage across the resistor or current through the resistor.

c)Over a frequency range of 0-100 kilohertz,the measurement circuitry is to have a frequency response – ratio of indicated to actual value of current – equal to the ratio of the impedance of a 1500-ohm resistor shunted by a 0.15-microfarad capacitor to 1500 ohms.At an indication of 0.5 milliampere,the measurement is not to have an error of more than 5 percent at 60 hertz.

d)Unless the meter is being used to measure leakage from one part of an appliance to another,the meter is to be connected between the accessible parts and the grounded supply conductor.

29.6 A sample of the appliance is to be tested for leakage current in the as-received condition,without prior energization except as may occur as part of the production line testing,but with the grounding conductor,if any,open at the attachment plug.The supply voltage is to be adjusted to the applicable values specified in 33.1.8.The test sequence,with reference to the measuring circuit,Figure 29.1,is to be as follows.

a)With the switch S1 open,the appliance is to be connected to the measuring circuit.Leakage current is to be measured using both positions of switch S2,and with the appliance switching devices in all their normal operating positions.

b)Switch S1 is then to be closed,energizing the appliance,and within 5 seconds,the leakage current is to be measured using both positions of switch S2 and with the appliance switching devices in all their normal operating positions.

c)The leakage current is to be monitored until thermal stabilization.Both positions of switch S2 are to be used in determining this measurement.Thermal stabilization is to be obtained by operation as in the normal temperature test.

29.7 Normally the complete leakage current test,as described in 29.6,is to be conducted without interruption for other tests.However,with the concurrence of those concerned,the leakage current test may be interrupted to conduct other nondestructive tests.

Figure 29.1

Leakage current measurement circuits

30 Continuity of Grounding Circuit Test

30.1 The resistance shall be not more than 0.1 ohm between any point required to be grounded,as specified in 25.1.2,and:

a)For an appliance intended for permanent electrical connection,the point on the enclosure at which the power-supply system will be connected.

b)For a cord-connected appliance employing a grounding conductor in the cord,the point to which the grounding conductor of the power-supply cord is connected.

30.2 The resistance may be determined by any convenient method except that if unacceptable results are obtained,either a direct or alternating current equal to the current rating of the maximum-current-rated branch-circuit overcurrent-protective device that may be employed with the appliance is to be passed from the equipment grounding terminal or the point of attachment of the wiring system to the dead metal part,and the resulting drop in potential is to be measured between these two points.The resistance in ohms is determined by dividing the drop in potential in volts by the current in amperes passing between the two points.

31 Starting Current Test

31.1 When operated as described in 31.3,an appliance shall start and operate normally without:

a)Tripping an overload protector provided as part of the appliance;or

b)Opening the fuse,when connected to a circuit protected by a fuse as described in 31.2.

31.2 The fuse mentioned in 31.1(b)is to be other than a time-delay type.The current rating of the fuse is to be equal to the current rating of the supply circuit of the lowest rating to which the appliance is intended to be connected.

Exception:A time-delay fuse may be employed,provided that the appliance is marked in accordance with 55.1.2 and:

- The construction of the appliance or the nature of its usage is such that it is likely to be used continually on the same branch circuit after installation – for example,a window fan,an attic ventilator,or the like;or

- The appliance is of the household type that would normally be used on a 15- or 20- ampere branch circuit.

31.3 To determine whether an appliance complies with the requirements in 31.1.the appliance is to be connected to a power-supply circuit protected by a fuse as specified in 31.2 or in the exception to 31.2,whichever applies.The appliance is to be at room temperature at the beginning of the test.The appliance is to be started three times without tripping an overload protector provided as part of the appliance,or opening the fuse protecting the supply circuit.Each start of the appliance is to be made under conditions representing the beginning of normal operation-the beginning of the normal operating cycle in the case of an automatic appliance.The motor of the appliance is to be allowed to come to full speed after each start,and to come to rest between successive starts.

31.4 An appliance employing a general use receptacle shall be loaded to the marked rating of the receptacle outlet.See 18.18

32 Input Test

32.1 The current input to an appliance shall be not more than 110 percent of the rated value when the appliance is operated under conditions of intended service,when connected to a power-supply circuit of maximum rated voltage and rated frequency.

33 Temperature Test

33.1 All fans

33.1.1 When tested as described in 33.1.3 – 33.2.2,an appliance shall not:

a)Attain constant temperatures at any point on the fan sufficiently high to cause a risk of fire;

b)Cause deterioration of any materials employed in the appliance;or

c)Have constant temperature rises at specific points (particularly those surfaces that may be contacted by the user with the fan operating as intended) more than those specified in Table 33.1.(See 33.1.10.)

33.1.2 A thermal protective device shall not operate during the temperature test.See 21.6.

31.1.3 Coil and winding temperatures are to be measured by thermocouples located on exposed surfaces,except the change-in-resistance method may be used for a coil that is inaccessible for mounting of thermocouples,such as a coil:

a)Immersed in sealing compound;

b) Wrapped with thermal insulation;or

c) Wrapped with more than two layers of material such as cotton,paper,or rayon having a total thickness of more than 0.8 mm (1/32 inch).

In an alternating-current motor having a frame diameter of 177.80 mm (7 inches) or less,the thermocouple is to be mounted on the integrally-applied (enamel) insulation of the windings.

33.1.4 Thermocouples are to consist of wires not larger than No.24 AWG.When thermocouples are used in the determination of temperatures in connection with the heating of electrical devices,it is common practice to employ thermocouples consisting of No.30 AWG iron and constantan wires and a potentiometer-type indicating instrument;and such equipment is to be used whenever referee temperature measurements by thermocouples are necessary.

Table 33.1

Maximum acceptable temperature rises

| Materials and components | C(Degrees) | F(degrees) |
| --- | --- | --- |
| All Fans - Varnished-cloth insulation - Fuses - Fiber employed as electrical insulation - Wood and other combustible material - Any point on or within a terminal box of a permanently-wired appliance - A surface upon which a stationary appliance may be mounted in service,and surfaces that may be adjacent to the appliance when so mounted - Class A insulation systems on coil windings of an a-c motor having a diameter of more than 177.80 mm (7 inches),of a d-c motor and a universal motor,and on a reactor - In an open motor Thermocouple method Resistance method - In a totally enclosed motor Thermocouple method Resistance method 8. Class A insulation systems on coil windings of an a-c motor having a diameter of 177.80 mm(7 inches),or less (not including a universal motor) - In an open motor Thermocouple or resistance method - In a totally enclosed motor Thermocouple or resistance method 9. Class B insulation systems on coil windings of an a-c motor having a diameter of more than 177.80 mm (7 inches) ,of a d-c motor and of a universal motor | 60 65 65 65 65g 65 65a 75 70 80 75a 80 | 108 117 117 117 117 117 117a 135 126 144 135a 144 |

Table 33.1 Continued

| Materials and components | C(Degrees) | F(Degrees) |
| --- | --- | --- |
| - In an open motor Thermocouple method Resistance method - In a totally enclosed motor Thermocouple method Resistance method 10. Class B insulation systems on coil windings of an a-c motor having a diameter of 177.80 mm (7 inches)or less (not including a universal motor) and on a vibrator coil A. In an open motor and on a vibrator coil Thermocouple or resistance method - In a totally enclosed motor Thermocouple or resistance method 11. Phenolic composition employed as electrical insulation or as a part of the deterioration of which would result in a risk of fire,electric shock,or injury to persons 12. Rubber ot thermoplastic-insulated wires and cords 13. Capacitors Electrolytic Other types - Sealing compound - Class 105 insulation systems on windings of a relay,a solenoid,and the like. Thermocouple method Resistance method - Class 130 insulation systems on windings of a relay,a solenoid,and the like Thermocouple method Resistance method - Lampholder screw shell,center contact,or other connecting device of aluminum or unplated copper - Lampholder body of thermosetting material (phenolic,urea,and the like) Ceiling Insert Fan/Light Combinations - Points of fixture support - Screen,shield,or recessed housing in contact with insulation Surfaces Subject to Contact h - Handles or knobs that are grasped for lifting,carrying,or holding: Metallic Nonmetallic - Handles or knobs that may be contacted but do not involve lifting,carrying,or holding when heated;and other surfaces that may be contacted during intended operation: Metallic Nonmetallic | 85a 95 90 100 95a 100 125b 35b,c,d 40e 65f less than melting point 40℃(72℉) 65a 85 85a 105 175 125 65 65 25 35 35 60 | 153a 171 162 180 171a 180 225b 63b,c,d 72e 117f 117a 153 153a 189 315 225 117 117 45 63 63 108 |
| a On the surface of an insulated coil where the temperature is affected by an external source of heat (integral lights or light kits of a fan are not considered to be external sources of heat),the temperature rise measured by a thermocouple may be higher than the specified maximum,if the temperature rise of the coil measured by the resistance method is not more than that specified in the table.The additional acceptable temperature rises above the values specified are: ltem Temperature Part A of item 7 15℃(27℉) Part A of item 8 5℃(9℉) 13 15℃(27℉) Part A of item 14 20℃(36℉) Part A of item 15 10℃(18℉) 16 20℃(36℉) b The temperature limitations on phenolic composition and on rubber and thermoplastic insulation do not apply to a compound that has been investigated and found to have acceptable heat-resistance properties. c Rubber-insulated conductors within a Class A insulated motor and rubber-insulated motor leads,and a rubber-insulated flexible cord entering a motor may be subjected to a temperature rise of more than 35℃(63℉),provided that an acceptable braid is employed on the conductors of other than a flexible cord.This does not apply to thermoplastic-insulated wires or cords. d A short length of rubber or thermoplastic-insulated flexible cord exposed to a temperature rise of more than 35℃(63℉),such as at terminals,is acceptable if supplementary heat-resistance insulation of acceptable dielectric strength is employed on the individual conductors of the cord to reduce the risk of deterioration of the conductor insulation,and if the strain-relief means does not depend upon that portion of the insulation subjected to the excessive temperature. e For an electrolytic capacitor that is integral with or attached to a motor,the temperature rise on insulating material integral with the capacitor enclosure shall not be more than 65℃(117℉). f A capacitor that operates at a temperature rise of more than 65℃(117℉) may be jubged on the basis of its marked temperature limit. g If the temperature rise is greater than 35℃(65℉),the fan shall be marked in accordance with 55.7.1 -55.7.3. h Based on a 25℃(77℉) ambient temperature. i This does not apply to the motor of a pedestal fan on which either no switches are mounted or on which no switches other than pull- chain type are mounted. |  |  |

33.1.5 If the change-in-resistance method is used,determination of the temperature rise of a winding is to be calculated by the following formula:

in which:

△t is the temperature rise in ℃;

R2 is the resistance of the coil in ohms at the end of test;

R1 is the resistance of the coil in ohms at the beginning of the test;

t1 is the ambient temperature in degrees C at the beginning of the test;

t2 is the ambient temperature in degrees C at the end of the test;and

K is 234.5 for copper and 225.0 for electrical conductor grade (EC) aluminum;values of the constant          for other conductors are to be determined.

33.1.6 If necessary,the value of R at shutdown may be determined by taking several resistance measurements at short intervals,beginning as quickly as possible after shutdown.A curve of the resistance values and the time is to be plotted and extrapolated to give the value of R shutdown.

33.1.7 All values for temperature rises in Table 33.1 are based on an assumed ambient temperature of 25℃(77℉).However,tests may be conducted at any ambient temperature within the range of 10-40℃(50-104℉).

33.1.8 For the temperature test,the test voltage for an appliance is to be as specified in Table 28.1.If the appliance has a single frequency rating,the test is to be conducted at that frequency.An appliance rated 25-50 hertz or 50-60 hertz is to be tested using a 60-hertz supply.

33.1.9 The appliance is to be operated under each condition of normal service.For a multispeed appliance,this includes operation at each speed and,for a reversible appliance,it includes operation in each direction of rotation.If a reversible appliance continues to rotate in the same direction,but at a slower speed,when the reversing switch is thrown,the requirement applies at the lower speed as well as at the normal speed.The test is to be continued until temperatures have become constant.

33.1.10 A temperature is considered to be constant when three successive readings taken at intervals of

10 percent of the previously elapsed duration of the test,but not less than 5 minutes,indicate no further increase.

33.1.11 An appliance employing a general use receptacle shall be loaded to the marked rating of the receptacle.The maximum length of the supply cord is to be used for the Temperature Test.

33.1.12 For fans rated 12 Amps or less,and that employ wiring terminals for field wiring connections,the Temperature Test is to be conducted with supply wiring sized for the ampacity of the fan.When the fan is rated more than 12 Amps,the fan is to be tested with supply wiring sized 125 percent of the rating of the fan.

33.2 Fans for use in cooking areas and oven ventilators

33.2.1 The temperature test for a fan intended for use over an eye-level range oven is to be conducted with the appliance mounted above a heat source as described in 32.2.2 and illustrated in Figure 33.1,in accordance with the manufacturer’s instructions.The test is to be conducted with the appliance and heat source in a 2-sided-right side and rear-alcove of 9.525-mm (3/8 inches) thick plywood.The sides of the alcove are to be painted black and are to extend at least 609.60 mm (2 feet) beyond the outermost edges of the assembly.The appliance and heat source are to be as close to the side and back of the alcove as their configuration permits.The test is to be conducted first with the fan on and with only the bake element energized with the thermostat set to give a temperature of 246℃(475℉) with the door of the heat source closed.The test is then repeated with the heat source set for broiling and the door open 101.60 mm (4 inches).Both of these tests are then to be repeated with the fan off.

32.2.2 The heat source illustrated in Figure 33.1 is to be constructed of sheet metal having an average thickness of 0.81 mm(0.032 inch).The thermal insulation is to weigh 16.02 kilograms per cubic meter (1 pound per cubic feet).The heat source is to be approximately 381 mm (15 inches) deep,front to back,including the 25.4-mm (1-inch) thick door.The door is to be hinged on the left side and may be closed or opened as much as 101.60 mm (4 inches).The elements are to consist of coiled resistance wire embedded in fused magnesium oxide completely enclosed in nickel-alloy stainless steel.The heat from the source is to be controlled with a thermostat in which the bulb and capillary tube are located at the top of the heat source,toward the side.

Figure 33.1

Heat source for temperature tests of fans for use in cooking areas and oven ventilators

33.3 Controllers

33.3.1 For the temperature test,a separate controller – that is,a controller that is not a physical part of the appliance – that is intended for installation in a wall is to be mounted as follows.The controller is to be secured inside its own enclosure,if provided.Otherwise it is to be installed inside the smallest standard flush-type outlet box that will accommodate it,and the box is to be mounted in a simulated wall section as illustrated in Figure 33.2.

Figure 33.2

Method of mounting outlet box

34 Dielectric Voltage withstand Test

34.1 An appliance shall withstand for 1 minute without breakdown the application of a 60-hertz essentially sinusoidal potential between live parts and dead metal parts,and between live parts of oppo-site polarity of a capacitor used for power-factor correction or for radio-interference elimination with the appliance at the maximum operating temperature reached in intended use.The test potential is to be:

a) One thousand volts for an appliance employing a motor rated 373 watts(1/2 horsepower output) or less and rated 250 volts or less.See 24.1.4.

b) One thousand volts plus twice the rated voltage for an appliance employing a motor rated more than 373 watts (1/2 horsepower output) or more than 250 volts.See 24.1.4.

c) Five hundred volts for an appliance operating at 50 volts or less.See 24.1.4.

34.2 To determine whether an appliance complies with the requirements in 34.1,it is to be tested using a 500- volt-ampere or larger capacity transformer,the output voltage of which can be varied.The applied potential is to be increased from zero to the required test value and is to be held at that level for 1 minute.The increase in applied potential is to be at a substantially uniform rate as rapid as is consistent with correct indication of its value by a voltmeter.

34.2.1 The test equipment for conducting the dielectric voltage-withstand test is to be adjusted for sensitivity such that when a resistor of 120,000 ohms is connected across the output,the test equipment does not indicate unacceptable performance for any output voltage less than the specified test voltage,and the test equipment does indicate unacceptable performance for any output voltage equal to or greater than the specified test value.The resistance of the calibrating resistor is to be adjusted as close to 120,000 ohms as instrument accuracy can provide,and not more than 120,000 ohms.

Exception No.1: The sensitivity of the test equipment shall not be reduced,and a lower value of calibrating resistance shall not be used,unless testing an appliance intended to be permanently wired.

Exception No.2: The sensitivity of the test equipment shall not be increased,and a higher value of calibrating resistance shall not be used,unless agreeable to those concerned.

34.3 An appliance employing a reversible shaded-pole motor shall withstand for 1 minute the application of a 60-hertz essentially sinusoidal potential:

a) Between the stator winding and the shading coils;or

b) Between the shading coils and the stator core,with the appliance at the temperature reached in intended use.

The value of the test potential is to be as specified in 34.1.

35 Water Spray Test

35.1 General

35.1.1 The following appliances shall comply with the applicable water spray test requirements as specified in this section:

a) Any appliance intended for installation in a window opening (See 56.7 and 56.8)

b) A roof ventilator;

c) An attic fan;

Exception:When an attic fan has instructions that comply with 57.2.8,the fan is not required to be subjected to the tests described in this section.

- A wall-insert fan,other than a wall-insert fan intended for use over a bathtub or shower and tested in accordance with Section 127,Performance;

Exception:When a wall-insert fan is intended for commercial or industrial use and is marked in accordance with 55.5.4 and 55.5.6,or when the fan is intended for mounting in an interior wall only and complies with 55.5.5,the fan is not required to be subjected to the tests described in this section.

- An in-wall fan;

- Any appliance that is intended for outdoor use or in any location that includes direct contact with water or rain;

- A nonresidential cord-connected fan that is not marked in accordance with 56.5,and that is not intended to be fastened in place;

- A dryer type fan;and

Exception:When the dryer type fan already complies with 141.1.1,the water spray test is not required.

i) A damp location fan for use in cooking areas.

Exception:When it is obvious from the construction or installation that an appliance would exclude the water spray,the water spray test is not required to be conducted.

35.1.2 Compliance with the test shall be determined by:

a) A visual inspection to determine that there is no wetting of uninsulated live parts or film-coated wire – other than motor windings – and no accumulation of water in wiring compartment or channel.The accumulation of water droplets may be determined to be acceptable provided the fan complies with the tests in (b) and (c).A drain hole may be provided to reduce the accumulation of water above a level that would result in the wetting of an electrical part or wiring compartment.

b) A leakage current test for 120 volt cord connected appliances (see 35.4.1 – 35.4.6) or an insulation resistance test for all other appliances (see 35.5.1).

c) A dielectric voltage withstand test (see 35.6.1).

35.2 Test preparation

35.2.1 The water spray test apparatus is to consist of three spray heads mounted in a water supply pipe rack as shown in Figure 35.1.Spray heads are to be constructed in accordance with the details shown in Figure 35.2.The water pressure for all tests is to be maintained at 34.48 kPa (5 psi)at each spray head.The distance between the center nozzle and the appliance is to be approximately 1.52 m(5 feet).The spray is to be vertical toward the louvers or other openings nearest current-carrying parts.

Figure 35.1

Spray head pipe rack

Figure 35.2

Spray head assembly

35.2.2 The water to be used for the test is to have its resistivity adjusted before the test is started to 3500 ohm-centimeters ±5 percent when measured at 25℃(77℉).At the conclusion of the test,the resistivity of the water is to be not less than 3200 ohm-centimeters or more than 3800 ohm-centimeters at 25℃.

35.3 Procedure

35.3.1 The appliance is to be mounted in accordance with the installation instructions.For items of installation not covered by the instructions.the most severe method of test installation is to be used.

35.3.2 The appliance is to operate during the exposure if it:

a) Is intended to draw air in rather than expel it ;or

b) Has louvers that open only when the appliance is operating.

Other appliances are not to operate during the water spray.

35.3.3 The appliance is to be subjected to a water spray for 4 hours in the direction most likely to cause water to enter.

35.3.4 A box fan that is marked in accordance with 56.7 is to be subjected to a water spray for 2 hours in the direction most likely to cause water to enter.The fan is to be energized for 1 hour and then de-energized for 1 hour during the water spray.

33.3.5 Before conducting a leakage current measurement with the circuit in Figure 35.3,the appliance is to be disconnected from the receptacle,and it should be determined by closing S1,utilizing both positions of S2,and observing the leakage current at the meter,that the stray leakage current are negligible.

Figure 35.3

Leakage current measurement circuit for water spray test

35.4 120 volt cord-connected appliance

35.4.1 The water spray test is to be conducted with the switch in the position.For a multispeed appliance,the on position of the switch is to be the low-speed setting.

35.4.2 The leakage current is to be measured using the test methods and equipment described in 29.2 – 29.5,except that the circuit shown in Figure 35.3 is to be used.The leakage current is to be measured with switches S1 and S2 in all possible positions.

35.4.3 During exposure to the water spray described in 35.2.1,the leakage current is to be monitored using both positions of S2.The leakage current shall be no more than 2.5 milliamperes.

35.4.4 Immediately upon cessation of the water spray,the appliance is to be turned off by opening switch S1 unless it was in the open position during the exposure to the water.The leakage current shall be no more than 2.5 milliamperes.

35.4.5 After the 30-minute monitoring period,the leakage current is to be measured with switch S1 open and using both positions of switch S2.The leakage current shall be no more than 0.5 milliampere.

35.4.6 Immediately after each of the leakage current measurements,a dielectric voltage withstand test is to be conducted as described in 35.6.1.

35.5 Other than 120 volt cord connected appliance

35.5.1 Immediately after the water spray test,the insulation resistance measured between live parts and interconnected dead metal parts of an appliance shall be 50,000 ohms or more.After the insulation resistance measurement,the appliance shall comply with the dielectric voltage withstand test described in 35.6.1.

35.6 All appliances

35.6.1 Within 1 minute of cessation of the water spray and also 30 minutes after cessation of the water spray,a dielectric voltage withstand test shall be conducted in accordance with Section 34,Dielectric Voltage Withstand Test,except that the duration of the applied potential is to be 15 seconds for the test conducted within 1 minute of cessation of the water spray test.

35A Hosedown Test

35A.1 When required by 55.1.8,the hosedown test shall be conducted on a fan.

35a.2 At the conclusion of the test described in 35A.3 – 35A.5,a fan shall have no standing water inside the enclosure and there shall be no water on uninsulated live parts or on film-coated wire,other than motor windings.

35A.3 For a permanently installed fan that is connected to conduit during installation,a conduit is to be installed on the enclosure to equalize internal and external pressures as required,but it is not to serve as a drain.No sealing compound other than that normally provided by the manufacturer is to be used.The enclosure interior is to be dry.

35A.4 The enclosure and its external mechanisms are then to be sprayed by water from a hose having a 25.4-mm(1-inch) inside diameter nozzle that delivers at least 246 L.(65 gallons) of water per minute.The water stream is to be directed at the fan from a distance of 3-3.7m(10 – 12 feet) and is to be moved along the enclosure or surface at a minmum rate of 1.6 cm/sec (4 inches per second).

35A.5 For an enclosure having a test length – height plus width plus depth dimension – of 1.9 m(75 inches) or less,the duration of the water stream contact with the enclosure is to be 5 minutes.For an enclosure having a test length exceeding 1.9m (75 inches),the duration of water stream contact in minutes is to be 2.6 times the test length measured in meters (the test length measured in inches divided by 15).

36 Locked Rotor Test

36.1 An impedance-protected motor used in a wall-insert fan,ceiling-insert fan,a fan intended for use in cooking areas,or a fan in which the motor is subjected to conditions such as restricted ventilation or proximity to an external source of heat,shall comply with the requirements in 36.4 and 36.5.

36.1.1 For fans rated 12 Amps or less,and that employ wiring terminals for field wiring connections,the Locked Rotor Test is to be conducted with supply wiring sized for the ampacity of the fan.When the fan is rated more than 12 Amps,the fan is to be tested with supply wiring sized 125 percent of the rating of the fan.

36.2 The fan is to be installed or placed in its intended position with the rotor of the fan motor locked and energized as described in the Locked-Rotor Temperature Test in UL2111,the Standard for Overheating Protection for Motors.The temperature is to be determined after thermal equilibrium is attained.

36.3 In determining compliance with the requirements specified in 36.2,motor temperature readings are to be taken as follows:

a) For a totally enclosed motor,the temperature is to be measured by means of thermocouples on the enclosure.

b) For a motor with an encapsulated coil complying with the applicable requirements specified in the Standard for Electric Motors,UL 1004,the motor is to be considered a totally enclosed motor and the temperature is to be measured as described in (a).

c) For an open motor,the temperature is to be measured by means of thermocouples on the integrally applied insulation of the winding – that is ,under the coil wrap.

36.4 Under locked-rotor conditions:

a)The stabilized temperature of an impedance-protected motor shall not exceed the temperature obtained when the motor was tested in the open in accordance with the Locked-Rotor Test in UL 2111,the Standard for Overheating Protection for Motors;or

b)The motor shall comply with the requirements in 36.5,Exception No.2 to 20.1(a),or the Exception to 20.1(b).

36.5 When the stabilized temperature of the impedance-protedted motor installed in the fan exceeds the temperature of the motor tested in the open in accordance with UL 2111,the Standard for Overheating Protection for Motors or if the motor was never so tested,the rotor is to remain locked for 18 days under the conditions specified for the Locked Rotor Test in UL 2111,except the motor is to be installed in the fan as specified in 36.2.During the initial 72 hours,the motor temperature shall not exceed the applicable temperature limits specified in UL 2111,or 200℃(392℉) for impedance-protected Type F fan motors.During and after the 18-day period,the motor shall comply with the Locked Rotor Test requirements in UL 2111.

36.6 Under locked rotor conditions,the temperature of a thermally protected motor shall not exceed the requirements in UL 2111,the Standard for Overheating Protection for Motors.

37 Humidity Conditioning Test

37.1 After conditioning as specified in 37.2;

a) The appliance shall comply with the dielectric voltage withstand requirements in 34.1;

b) For a cord-connected appliance rated 240 volts or less,the leakage current shall be not more than the applicable value in 29.1;and

c) For an appliance other than as mentioned in (b),the insulation resistance between current-carrying parts and exposed dead metal parts shall be not less than 50,000 ohms.

37.2 For the conditioning mentioned in 37.1,a sample of the appliance is to be heated to b temperature just above 34℃(93℉) to reduce the likelihood of condensation of moisture during conditioning.The heated sample is then to be placed in the humidity chamber and conditioned for 48 hours in air having a relative humidity of 88±2 percent and a temperature of 32±2℃(90±4℉).Following the conditioning:

a) A cord-connected appliance rated for a nominal 240-volt supply or less is to be tested in the unenergized condition as specified in 29.6(a).The sample is then to be energized and tested as specified in 29.6(b) and (c).The test is to be discontinued when the leakage current stabilizes or decreases.

b) An appliance other than as mentioned in (a) is to have an insulation resistance of 50,000 ohms or more between live parts and interconnected dead metal parts.

c) An appliance shall comply with the dielectric voltage withstand requirements in 34.1.

37.3 Insulation resistance is to be measured by means of a high-resistance voltmeter using a 250-volt,direct-current circuit.

38 Strain Relief Test

38.1 The strain relief means provided on a flexible cord shall withstand for 1 minute without displacement a direct pull of 155.68 N (35 pounds) applied to thecord with the connections within the appliance disconnected.The strain relief is not acceptable if,at the point of disconnection of the conductors,there is sich movement of te cord,as to indicate that sress on the connections would have resulted.

Exception:The strain relief means provided in a through cord switch may be in accordance with the requirements of 38.3.

38.2 A 16-kg(35-pound)weight is to be suspended from the cord and supported by the appliance so that the strain-relief means will be stressed from any angle the construction of the appliance permits.

38.3 The strain relief means provided in a through cord switch shall withstand for 1 minute a direct pull of 133.44N(30 pounds).The strain relief is not acceptable if a conductor is detached from a terminal or an uninsulated conductor of the cord is exposed.

38.4 For the construction mentioned in 14.2.3,six samples of the clamp that have been secured to the cord in the intended manner are to be used.Three samples are to be subjected to the dielectric voltage withstand test specified in Section 34,Dielectric Voltage Withstand Test,and shall them comply with the strain-relief test specified in 38.1 in the as-received condition. Three samples shall comply with the requirements specified in 38.1 after being subjected to the following procedures:

a) The samples are to be placed for 168 hours in a forced-draft air-circulating oven maintained at a temperature of 70℃(158℉) or 10℃(50℉) higher than the temperature recorded on the clamp during the Temperature Test,Section 33,whichever is greater.

b) The samples are then to be subjected to the dielectric voltage withstand test specified in Section 34,Dielectric Voltage Withstand Test,with the value of the applied potential based on the rating of the appliance.The potential is to be applied between conductors,and the potential is also to be applied between the clamp and all conductors spliced together.

c) The conditioned samples then are to be cooled at room temperature.

38A Push Back Relief Test

38A.1 With reference to 14.2.4,a cord-connected fan shall be tested in accordance with 38A.2 without occurrence of any of the following conditions:

- Mechanical damage to the supply cord or lead;

- Exposure of the supply cord or lead to a temperature higher than that for which it is rated;

- Reduction of spacings (such as to a metal strain-relief clamp) below the minimum required values;or

- Damage to internal connections or components.

38A.2 The supply cord or lead is to be held 25 mm(1 inch) from the point where the cord or lead emerges from the product and is then to be pushed back into the product.When a removable bushing which extends further than 25 mm (1 inch) is present,it is to be removed prior to the test.When the bushing is an integral part of the cord,the test is to be carried out by holding the bushing.The cord or lead is to be cord into the product exceeds 27 N (6 pounds-force).The supply cord or lead within the product is to be manipulated to determine compliance with 38A.1.

39 Unguarded Impeller Tests

39.1 An impeller of a freestanding fan not required to be guarded shall not break,crack,or chip when operated for 1 hour connected to a supply voltage of 130 percent of the rated supply voltage.The test is to be conducted before and after conditioning as described in 39.2.

39.2 An impeller not required to be guarded is to be placed in an air-circulating oven maintained at 70℃(158℉) for 7 hours.After removal,the part is to be tested as described in Section 39,Unguarded Fan Blade Tests;Section 42,Impact Test on Guards;and Section 46,Drop Test.The part shall not break.

39.3 A motor-driven impeller not required to be guarded shall be energy absorbent to the extent that a 3.2 mm (1/8 inch) diameter dry hardwood dowel does not break when thrust into a rotating blade.The fan is to be fixed in place and energized so that the impeller rotates at intended speed.The dowel is to be supported on a stable,stationary flat surface perpendicular to the plane of rotation.The surface is to have a straight edge located approximately 9.5 mm (3/8 inch)from the fan blade.The dowel is then to be suddenly thrust and retained by hand along an axis perpendicular to the plane of impeller rotation into the blade.This procedure is to be repeated at different points on the impeller (blade and hub) with the impeller rotating at all intended speeds and from in front of and behind the impeller in order to include the most severe condition.

40 Tests of Swithes and Controls

40.1 Overload

40.1.1 A switch or other device that controls a motor and that has not been previously investigated for this purpose shall perform acceptably when subjected to an overload test consisting of 50 cycles of operation,making and breaking the locked-rotor current of the motor.There shall be no electrical or mechanical breakdown of the device or pitting or burning of the contacts that could impair intended operation as a result of the tests,and the fuse in the grounding connection shall not open.

Exception:A switch or other device interlocked so that it will not have to break the locked-rotor current of the motor need not comply with the requirement.

40.1.2 In a test to determine whether a switch or other control device complies with the requirements in 40.1.1,the appliance is to be connected to a grounded supply circuit of rated frequency and maximum rated voltage in accordance with 33.1.8,with the rotor of the motor locked in position.During the test,exposed dead metal parts of the appliance are to be connected to ground through a 3 ampere plug fuse,and the connection is to be such that any single-pole,current-rupturing device is connected in the ungrounded conductor of the supply circuit.If the appliance is intended for use on direct current,or on direct current as well as on alternating current,the exposed dead metal parts are to be connected so as to be positive with respect to a single-pole,current-rupturing control device.The device is to be operated at a rate of not more than 10 cycles per minute,except that a faster rate of operation may be employed if agreeable to all concerned.

40.2 Reversing

40.2.1 A motor-reversing switch shall withstand a test consisting of 1000 cycles of operation as described in 40.2.2.There shall be no electrical or mechanical breakdown of the switch,nor pitting or burning of the contacts that could impair intended operation.

Exception:A reversing switch that reverses the direction of rotation by changing the connections between shading coils that are insulated from the main windings need not comply with this requirement.

40.2.2 To determine whether a switch complies with the requirements in 40.2.1,the appliance is to be connected to a circuit of maximum rated voltage,Each cycle of operation is to consist of throwing the switch to the position in which the fan blades rotate in one direction,allowing the blades to come to full operating speed in that direction,then – without pause in any intermediate off position unless the switch will not function otherwise – throwing the switch to the position in which the rotation of the blades is reversed,allowing the rotation to attain normal operating speed in that direction,and then reversing the direction of rotation again by throwing the switch to the initial on position.

41 Static Load Test For Mounting Means

41.1 A cord-connected wall-surface-mounted appliance,when subjected to the test specified in 41.2,shall comply with the following:

a) The security of the attachment of the appliance to the wall shall not be adversely affected;

b) There shall be no evidence of a risk of fire or electric shock;

c) The insulation resistance between live and dead-metal parts shall not be less than 50,000 ohms;and

d) The appliance shall withstand for 1 minute without breakdown the application of a 60-hertz essentially sinusoidal potential of 1000 volts between live and dead-metal parts.

41.2 An appliance as mentioned in 41.1 is to be mounted in the intended manner and subjected to a static load equal to three times the weight of the appliance,for 1 minute.The load is to be applied so as to transmit the maximum amount of stress to the mounting means.

42 Impact Test on Guards

42.1 A guard,the malfunction of which might result in a risk of injury to persons,that is subject to impact shall withstand the impact test described in 42.2 and 42.3.

42.2 An appliance is to be subjected to an impact of 6.67 N (1.5 foot-pounds) on any furface that is exposed to a blow during intended use.Only one impact is to be applied at a given point.The impact is to be produced by dropping a steel sphere,50.8 mm (2 inches) in diameter and weighing approximately 0.54 kg(1.18 pounds),from a height of 381 mm(15 inches).For surfaces other than the top of an enclosure,the steel sphere is to be suspended by a cord and allowed to swing as a pendulum,dropping through a vertical distance of 381 mm.For the test on a freestanding fan,the fan is to stand in its intended operating position without restraint.

42.3 Following the impact test,the probe illustrated in Figure 8.1 is to be used to determine whether a portion of an impeller that can cause risk of injury to persons is exposed.

42.4 Deformation of a guard or detachment of a guard or portion of a guard during the impact test is acceptable if the part can readily be restored to its original shape or replaced in the intended manner.After restoration of the guard,the probe illustrated in Figure 8.1 shall not contact a portion of an impeller that can cause risk of injury to persons when inserted in any opening of the guard.

42.5 When the guard also serves as an enclosure,the guard shall be subjected to the impact test specified in 42.1 – 42.4 and shall withstand the impact without occurrence of any of the following:

a) Making uninsulated live parts or film-coated wire accessible to contact by the probe specified in Table 9.1,applied as indicated in Section 9,Accessibility of Live Parts;

b) Producing a condition that affects the mechanical performance of the equipment;or

c) Producing a condition that would increase the risk of an electric shock.

42.6 With reference to 42.5(b),cracking or denting of the enclosure shall not affect the function of any safety controls or constructional features such as thermostats,overload protective devices,waterseals,or strain relief.

43 Static Force Test on Guards

43.1 When a 88.96-N (20-pound) force is applied for 1 minute over a 50.8-mm (2-inch) diameter area to any part of the guard,spacings to a moving part that can cause injury to persons shall not be permanently reduced to the degree that the probe shown in Figure 8.1 will contact a portion of an impeller that can cause injury to persons when inserted in any opening in the guard.

Exception:If the 88.96-N force tips over a freestanding fan,the force to be employed it the value that would give the maximum deflection without tipover.The test is to be conducted with and without the fan operating and with any adjustments made to provide the greatest resistance to tipping.

44 Impeller Test for Freestanding Fans and window Fans

44.1 With reference to the requirement in 8.1.3,the following tests are to be conducted on nonmetallic impellers.A 6.35 mm(1/4 inch) diameter steel rod is to be pushed suddenly into the blade with the fan resting on the floor and operating at maximum speed and rated voltage.A test is to be made with the rod inserted near the hub,and a second sample is to be tested with the rod inserted 2/3 of the distance from the hub to the tip of the blade.The rod is to rest on the guard as it is inserted.A part of the blade is not to be thrown more than 1.52 m(5 feet) from the closest part of the base of the fan.

44.2 An impeller shall be secured to the shaft,with consideration given to such factors as the size and weight of the impeller,motor power,mounted position,and thrust direction and the likelihood of a risk of injury to persons if breakage occurs.A reversible fan shall not rely solely on friction between the impeller and a nut turned onto a threaded shaft.

45 Impeller Ignition Test

45.1 The requirements in this section are intended to investigate the risk of fire for fans as referenced in the exception to 7.3(f).

45.2 A polymeric impeller that is located outside a thermally protected motor on a fan that does not comply with 7.3 and 7.3(a) – (e) of the exception shall not ignite as a result of a locked rotor test conducted as described in 45.3.

Exception:If the impeller ignites,the results are considered acceptable if the cheesecloth does not ignite and there is no emission of flame beyond the fan enclosure during the following test on three additional fans:

- A double layer of cheesecloth is to completely cover each fan before the test;and

- The test described in 45.3 is to be repeated.

45.3 The motor thermal protector is to be shunted out of the motor winding so that the motor stays continually energized.The rotor is to be locked.The fan is to be positioned as intended in application and is to be energized in a room ambient temperature of 10 to 40℃(50 to 104℉) at the voltage indicated in Table 28.1.The fan is to be energized until ultimate results are observed,but no more than 18 days.This procedure is to be repeated on two additional fan samples.

45.4 The cheesecloth referenced in the exception to 45.2 is to be bleached,914.40 mm(36 inches) wide,28.22 – 30.24 meters per kilogram (14-15 yards per pound),and having what is known to the trade as a count of 32x28-that is ,along the two directions parallel to the threads,there are 13 threads per centimeter in one direction and 11 thread per centimeter in the other (32 threads per inch in one direction and 28 threads in the other).

45A Component Breakdown Test

45A.1 As required by 24C.1,a fan shall be subjected to the Component Breakdown Test as specified in 45A.2-45A.12.

45A.2 With reference to 45A.1,a risk of fire or electric shock is determined to exist when any of the following occur:

- Glowing,charring,or flaming of the cheesecloth or tissue paper as specified in 45A.6;

- Opening of the 3 Amp fuse specified in 45A.7;

- Emission of flame,sparks,or molten metal from the enclosure;

- Creation of any openings in the enclosure that results in accessibility of live parts,when judged in accordance with Accessibility of Live Parts,Section 9;and

- Loss of structural integrity to a degree that the equipment collapses or experiences displacement of parts that leads to short-circuiting or grounding of live parts.

45A.3 The circuit between any two terminals of a device is to be opened or shorted.Only one of the simulated fault conditions is to be imposed at a time.For a multi-terminal device,only two terminals are to be short-circuited at a time.Simulated circuits are not prohibited from being used,but when the tests performed on simulated circuits indicate damage to other parts of the fan to the extent that the safety of the fan is affected,the tests shall be repeated on the fan.

45A.4 Each test is to be conducted on a separate sample unless it is agreeable to those concerned that more than one test be conducted on the same sample.

45A.5 A part of the fan that is removed during routine operation or maintenance is to be omitted when it results in a more severe test,and the part is not:

- Required for the functioning of the equipment;and

- Exposed to view during intended operation.

Exception:The part is not required to be removed during testing when it can only be removed by the use of a tool.

45A.6 During these tests,the sample is to be placed on a softwood surface covered with white tissue paper,and a single layer of cheesecloth is to be draped loosely over the entire enclosure.

Exception No.1:Fans not having bottom openings are not required to be placed on a softwood surface covered with white tissue paper.

Exception No.2:When it is impractical to drape the entire enclosure,cheesecloth is to placed only over all ventilating openings.

45A.7 During each test,exposed dead-metal parts of the sample are to be connected to earth ground through a 3 Amp nontime-delay fuse.

45A.8 The supply circuit is to have branch circuit overcurrent protection,the size of which equals 125 percent of the input current rating (20-ampere minmum).When this value does not correspond with the standard rating of a fuse or a circuit breaker,the next higher standard device rating shall be used .The test voltage and frequency are to be adjusted to the maximum rated voltage of the fan.

Exception:When a marking on the fan or the manufacture’s literature indicates the use of branch circuit protection exceeding 125 percent of the input current,such protection shall be used.

45A.9 A fuse that is replaced during routine maintenance is to be defeated unless marked in accordance with 56.11.A fuse that is soldered in place,or is located such that it is accessible only to qualified service personnel,and marked in accordance with 56.11,and any other overcurrent protective device not subject to replacement during routine maintenance is to be left in the circuit.

45A.10 Each abnormal condition is to be conducted for 7 hours or until one or more of the following results are observed:

- A risk of fire or electric shock develops (see 45A.2);

- The branch-circuit fuse opens;

- The supplementary protective device opens;

- A minimum of one hour elapses,circuit conditions stabilize,and there is no further evidence of overheating of parts.

45A.11 The overheating of parts referred to in 45A.10(e) is to be detected by an indicator such as an odor,smoke,discoloration,cracking of materials,charring.flaming,glowing,arcing,changes in circuit current through the applied fault,or any similar phenomenon.

45A.12 When a fault condition is terminated by the opening of a circuit component as specified in 45A.10(d),the test is to be conducted two more times using new components for each test.