FM24C02/04/08/16

两线制串行EEPROM

FM24C02/04/08/16 是 2048/4096/8192/16384比特的串行电可擦除及可编程只读存储器，内部为 256/512/1024/2048个字节，每个字节8个比特，该芯片广泛应用于低电压及功耗的工商业领域。

FM24C02/04/08/16 provides 2048/4096/8192/16348 bits of serial electricslly erasable and programmable read-only memory(EEPROM)organized as 256/512/1024/2048 words of 8 bits esch.The device is optimized fot use in many industrial and commercial applications where low-power and low-voltage operations are essential.

产品特点                              FERATURES

- 工作电压范围:2.7 v -5.5 v                  Vcc=2.7v to 5.5v

- 容量：256x8bit,512x8bit,1024x8bit,2048x8bit  lnternally Organized 2K,4K,8K,16K

- 两线串行接口                            2-wire Serial lnterface

- I2C数据传输协议                         Bi-directional Date Transfer Protocol

- 输入脚经施密特触发器滤波抑制噪声        Schmidt Trigger Filtered lnputs for Noise Suppression

- 100KHz(2.7v)和400KHz(5v)时钟兼容      100KHz(2.7v) and 400KHz(5v)Compatibility

- 写保护脚可使高地址的8k存贮单元硬件写保护Write Protect Pin for Hardware Data Protection

- 8-byte(2K),16-byte(4K,8K,16K)页写方式    8-byte (2K),16-byte (4K,8K,16K)Page Write Modes

- 允许只写部分页                          Partial Page Writes are Allowed

- 内部擦写时钟决定擦/写周期（＜10ms）     Self-timed Write Cycle(10ms max)

- 可靠性高                                High-reliability

- 允许100万次擦写                        Endurance:1 Million Write Cyclres

- 数据可保持100年                        Data Retention:100Years

引脚排列表      PIN ASSIGNMENITS

A0                                              VCC

FM24C02/04/

08/16

A1                                              WP

A2                                              SCL

GND                                            SDA

引脚功能表       PIN  ASSIGNMETS

| 管脚 Pin | 符号 Symbol | 说明 Description |
| --- | --- | --- |
| 1~3 | A0~A2 | 地址脚 Address Input |
| 4 | GND | 地 Ground |
| 5 | SDA | 串行数据 Serial Data |
| 6 | SCL | 时钟 Serial Clockl Input |
| 7 | WP | 写保护 Write Protect |
| 8 | VCC | 电源 Power Supply |

设备地址表    Device Address

| 2K 1 | 0 | 1 | 0 | A2 | A1 | A0 | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 4K 1 | 0 | 1 | 0 | A2 | A1 | P0 | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 8K 1 | 0 | 1 | 0 | A2 | P1 | P0 | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 16K 1 | 0 | 1 | 0 | P2 | P1 | P0 | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 电学特性 ELECTRICAL CHARACTERISICS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Applicable over recommended operating range from：TA1=-40oC to +85℃,Vcc=+2.7V to+5.5V, TAC=0℃ to+70℃, Vcc=+1.8V to+5.5V(unless otherwise noted). |  |  |  |  |  |  |  |  |  |
| 直流参数 DC CHARACTERISICS |  |  |  |  |  |  |  |  |  |

| 符号 Symbol | 参数 Parameter | 测试条件 Test conditions | 最小值 Min | 典型值 Typ | 最大值 Max | 单位 Unit |
| --- | --- | --- | --- | --- | --- | --- |
| # VCC | 电源电压 Supply |  | 2.7 |  | 5.5 | V |
| ICC | 电源电流 Supply Current Vcc=5v | Read at 100K |  | 0.4 | 1.0 | mA |
| ICC | 电源电流Supply Current Vcc=5v | Write at 100k |  | 2.0 | 3.0 | mA |
| ISB1 | 等待态电流Supply Current Vcc=2.7v | VIN=Vcc/Vss |  | 1.6 | 4.0 | μA |
| ISB2 | 等待态电流Supply Current Vcc=5v | VIN=Vcc/Vss |  | 8.0 | 18.0 | μΑ |
| ILI | 输入漏电流Input Leakage Current | VIN=Vcc/Vss |  | 0.10 | 3.0 | μΑ |
| ILO | 输出漏电流Out Leakage Current | VOUT=Vcc/Vss |  | 0.05 | 3.0 | V |
| VIL | 输入低电平Input Low Level |  | -0.6 |  | 0.3Vcc | V |
| VIH | 输入高电平Input High Level |  | 0.7Vcc |  | Vcc+0.5 | V |
| VOL2 | 输出低电平Output Low Level Vcc=3.0v | IOL=2.1mA |  |  | 0.4 | V |
| VOL1 | 输出低电平Output High Level Vcc=1.8v | IOL=0.15mA |  |  | 0.2 | V |

交流参数　　AC　CHARACTERISTICS

| 符号 Symbol | 参数 Parameter | 2.7V |  | 5.0V |  | 单位 Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 最小值 Min | 最大值 Max | 最小值 Min | 最大值 Max |  |
| fSCL | 输入时钟频率 Clock Frequency,SCL |  | 100 |  | 400 | KHz |
| TLOW | 时钟高电平 Clock Pulse Width High | 4.7 |  | 1.2 |  | μs |
| THIGH | 时钟低电平Clock Pulse Width Low | 4.0 |  | 0.6 |  | μs |
| TI | 噪音抑制时间Noise Suppression Time |  | 100 |  | 50 | ns |
| TAA | 时钟低至数据有效Clock Low to Data Out Valid | 0.1 | 4.5 | 0.1 | 0.9 | μs |
| TBUF | 两次指令间的总线空闲时间Time the bus must be free before a new transmission can start | 4.7 |  | 1.2 |  | μs |
| THD.STA | 起始位保持时间Start Hold Time | 4.0 |  | 0.6 |  | μs |
| TSU.STA | 起始位建立时间Start Setup Time | 4.7 |  | 0.6 |  | μs |
| THD.DAT | 数据保持时间Data In Hold Time | 0 |  | 0 |  | μs |
| TSU.DAT | 数据建立时间Data In Setup Time | 200 |  | 100 |  | ns |
| TR | 输入上升时间Input Rise Time |  | 1.0 |  | 0.3 | μs |
| TF | 输入上升时间Input Rise Time |  | 300 |  | 300 | ns |
| TSU.STO | 停止位建立时间Stop Setup Time | 4.7 |  | 0.6 |  | μs |
| TDH | 数据输出保持时间Data Out Hold Time | 100 |  | 50 |  | ns |
| TWR | 写周期Write Cycle Time |  | 10 |  | 10 | ms |
| Endurance | 5.0V,85℃ 字节操作模式 Byte Mode | 1M |  | 1M |  | Write Cycles |