ACT34	PRIMARY FEEDBACK OFF-LINE CONTROLLER

Features

- Lowest Total Cost Solution

- Primary-only feedback eliminates opto-isolator and many components

- Emitter Drive of NPN for Safe High Voltage Operation

- 0.3W Standby Power

- SoftEMITM for lowest EMI

- Very low line/load regulation

- Programmable Current Limit

- Short-circuit Foldback Current Limit

- Audible Noise Elimination

- Fast Response Current Mode PWM

- Tiny SOT23-6 package

- 65kHz or 100kHz Switching Frequency

GENERAL Description

AC/DC off-line power supply controller, emitter drive, primary feedback, and build-in SoftEMITM  technology, optimized for the lowest cost total solution with several patented technology by ACTIVE-SEMI to achieve high performance Constant Voltage control and Constant Current control.

APPLICATIONS

- Battery Chargers

- Power Adaptors

- Standby Power Supplies

- Appliances

- Universal Off-line Power Supplies

Figure 1. Typical Application Circuit

ORDERING INFORMATION

| DEVICE | FREQUENCY | PACKAGE | PINS |
| --- | --- | --- | --- |
| ACT34A | 65kHz | SOT23-6 | 6 |
| ACT34B | 100kHz | SOT23-6 | 6 |

PIN configuration

Top View

COMP

ACT34A

ACT34B

FB

V

ISET

SW

SOT23-6

G

Pin Description

| Pin NUMBER | Pin Name | Pin Description |
| --- | --- | --- |
| 1 | SW | Swich Ouput. Connect this pin to the emitter of the power MOSFET. |
| 2 | VDD | Supply Pin. Bypass this pin with a low ESR capacitor. |
| 3 | FB | Feedback Pin. The voltage at this pin is regulated to 1.28V. |
| 4 | GND | Ground. |
| 5 | ISET | Current Set Pin. Set the current limit with the current flow from this pin. |
| 6 | COMP | Compensation Pin. |

Absolute Maximum Ratings

(Note: Do not exceed these limits to prevent damage to the device. Exposure to absolute maximum rating conditions for long periods may affect device reliability.)

| parameter | value | unit |
| --- | --- | --- |
| VDD Supply Voltage | -0.3 to 16.5 | V |
| VDD Current | 30 | mA |
| ISET, FB Voltage | -0.3 to 6 | V |
| SW Voltage | -0.3 to 18 | V |
| Continuous SW Current | Internally limited | A |
| Maximum Power Dissipation | TBD | W |
| Junction to Ambient Thermal Resistance (JA) | TBD | °C/W |
| Operating Junction Temperature | -40 to 150 | °C |
| Storage Temperature | -55 to 150 | °C |

Electrical Characteristics

(VVDD = 12V, TJ = 25C unless otherwise specified)

| parameter | symbol | TEST CONDITIONS |  | Min | TYP | MAX | unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VDD Turn-On Voltage | VDDON | Rising edge |  |  | 13.6 |  | V |
| VDD Turn-Off Voltage | VDDOFF | Rising edge |  |  | 11.8 |  | V |
| Hysteresis |  |  |  |  | 1.8 |  | V |
| VDD Clamp Voltage | VDDCLAMP | IVDD = 25mA |  |  | 16 |  | V |
| Operation Current | IDD | VDD = 14V |  |  | 1 | 1.5 | mA |
| Quiescent Current | IQ | VDD = VDDON – 1V |  |  | 37 |  | uA |
| Switching Frequency | fSW | VFB = 1.25 | ACT34A | 50 | 65 | 85 | kHz |
|  |  |  | ACT34B | 85 | 100 | 125 |  |
| Short Frequency | fSC | VFB = 0V |  |  | 12 |  | kHz |
| Standby Frequency | fSB | VFB = 1.5V |  |  | 8.8 |  | kHz |
| FB Feedback Voltage | VFB |  |  | -2% | 1.25 | +2% | V |
| Maximum Duty Cycle | DMAX | ISW = 10mA |  | 67 | 75 | 83 | % |
| Minimum Duty Cycle | DMIN | ISW = 100mA |  |  | 3.5 |  | % |
| Current Limit | ILIM | RISET = 10k |  |  | 200 |  | mA |
| Current Limit Foldback Voltage | VFOLD |  |  |  | 0.32 |  | V |
| Switch On-Resistance | RSW | ISW = 0.05A |  |  | 1.8 |  | Ω |
| SW Rise Time |  | 1nF load, 15 pull-up |  |  | 30 |  | ns |
| SW Fall Time |  | 1nF load, 15 pull-up |  |  | 20 |  | ns |
| SW Switch Off Current |  | Switch in off-state, VSW = 13V |  |  | 1 | 10 | A |

Figure 2. Functional Block Diagram

### OPERATION

ACT34US is a novel off-line power supply controller based on emitter drive PWM operation. When controller is in the constant voltage operation, it uses a constant frequency, current mode scheme to provide excellent line and load regulation (refer to the block diagram).But the feedback voltage is extracted from the auxiliary voltage by sample-hole method based on the following theory. When controller is in the constant current operation, it uses a variable frequency, programmable current limit and base current compensation scheme to control output current.

1) Constant Voltage Control - Sampling Method

See figure below.

Discontinuous conduction: 300ns before large negative dV/dt.

VAUX = NAUX/NSEC*VSEC=NAUX/NSEC*(VOUT+VFSchottky).

In discontinuous conduction, ISWbottom=0. If we set regulation so that VAUX= VFB*(R1/R2+1), then

VOUT=[VFB*(R1/R2+1)*NSEC/NAUX-VFSchottky] with very little load regulation. The load regulation comes from VFSchottky's being smaller because of smaller current. So we want to sample VSEC before ISEC drops too close to zero.

2) Constant Current Control

In DCM operation, the input power of the controller is

0.5*Lm*Ipeak*Ipeak*f

lm, inductance of primary side;

Ipeak, peak current of primary side;

F, switching frequency.

ACT34US control the switching frequency by output voltage, then the output power of the controller is vout * Iout, and f = k*vout, and the constant current can be obtained,

Iout = 0.5*Lm*Ipeak*Ipeak*k*η

η, efficiency from input power to output power.

ISEC

Sample point

TMINOFF

3) Soft-EMI feature

Frequency dither , the switching frequency modulated by a low frequency 1k or so. This operation will help EMI design a lot; further more, the switching rate will also modulate, it will improve EMI feature.

4) Protection Feature

Built-in Vcc clamp circuit, the clamp has a high precision of 16.5V and 20mA current limited.

Over Voltage Protection, if FB is higher than 1.29, the frequency will decrease to about 10Khz.

PACKAGE OUTLINE

SOT23-6 PACKAGE OUTLINE AND DIMENSIONS

Active-Semi, Inc. reserves the right to modify the circuitry or specifications without notice. Users should evaluate each product to make sure that it is suitable for their applications. Active-Semi products are not intended or authorized for use as critical components in life-support devices or systems. Active-Semi, Inc. does not assume any liability arising out of the use of any product or circuit described in this datasheet, nor does it convey any patent license.

Active-Semi and its logo are trademarks of Active-Semi, Inc. For more information on this and other products, contact sales@active-semi.com or visit www.active-semi.com. For other inquiries, please send to:

44081 Old Warm Springs Blvd, Fremont, California 94538, USA