eltrend

偉詮電子股份有限公司

Weltrend Semiconductor Inc.

新竹市科學工業園區工業東九路24號2樓

TEL:03-5780241  FAX:03-5794278.5770419

KL0173C  SPECIFICATION

Pin Assignment

Fig.1  KL0173 Pin Assignment Diagram

Pin Description

| Pin no. | Name | Description |
| --- | --- | --- |
| 1 | PW_TM | output of IC test |
| 2 | VDD | VDD supply input |
| 3 | DRV | output of control power transistor |
| 4 | OSC2 | input of detect heat inductor’s voltage |
| 5 | OSC1 | input of detect heat inductor’s voltage |
| 6 | VCO | control DRV ’s duty cycle |
| 7 | F_BACK | output of control DRV ‘s duty cycle and DL ‘s voltage |
| 8 | TEST | input of IC test , please connect to GND |
| 9 | GND | ground |
| 10 | LINE | input of power frequency |
| 11 | RESET | output of power on reset |
| 12 | WARM | output (active low) - clock = fLINE÷64 , if no-load and at WARM state - low , if with-load and at WARM state - high , if at HEAT state |
| 13 | HEAT | output (active low) - clock = fLINE÷64 , if no-load and at HEAT state - low , if with-load and at HEAT state - high , if at WARM state |
| 14 | KEY | input of toggle WARM state / HEAT state ( at HEAT state when power on ) |
| 15 | DL | input of division voltage of F_BACK , decide with-load / no-load ‘s voltage point |
| 16 | DI | input of transfer voltage that load current be detected , - if DI > DL then enter with-load mode ii. if DI < DL then enter no-load mode |
| 17 | DV | input of detect voltage (for over voltage protection) , if DV > 5/6VDD then enter over voltage mode |
| 18 | DT_CIN | input of RC oscillator ( detect temp.) |
| 19 | DT_COUT | output of RC oscillator ( detect temp.) |
| 20 | SET_PT | input of setting WARM ’s temp. / HEAT ’s power |

KL0173 Block Diagram

在HEAT state時，當溫度超過210℃，停止加熱；直到溫度下降至180℃，延遲3秒恢復加熱。

在WARM state時，加熱功率為定值，當溫度超過＋5℃，停止加熱；

直到溫度下降至－5℃，延遲3秒恢復加熱。

一旦進入over voltage mode or no-load state，便停止加熱； 解除後則需延遲3秒才恢復加熱。

Over-voltage   if DV > 5/6VDD

No-load	    if DI < DL

With-load     if DI > DL

Recommended Operation Condition

| Item | Symbol | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Total Supply Voltage | VDD |  | 12 |  | V |
| Operating Temperature | Ta | 0 |  | 75 | ℃ |
| Stand-by current | IDD |  | 15 |  | mA |

Electrical Characteristics

( VDD=12V, Ta=25℃)

| Item | Symbol | Min. | Typ. | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Voltage range of F_BACK | VF_BACK | 0.2 |  | 0.7 | VDD |
| Input leakage current (註1) | IIL / IIH |  | ±1 |  | uA |
| Voltage range of SET_PT | VSET_PT | 0.0 |  | 1.0 | VDD |
| VCO sink current , VOL = 0.4V | IOL1 |  | 4 |  | mA |
| DRV source current , VOH = 11.6V | IOH2 |  | 11 |  | mA |
| DRV sink current , VOL = 0.4V | IOL2 |  | 7.5 |  | mA |
| Output source current , VOH = 11.6V (註2) | IOH3 |  | 0.8 |  | mA |
| Output sink current , VOL = 0.4V (註2) | IOL3 |  | 1.2 |  | mA |
| ESD (100pF , 1.5 KΩ) (註2) |  | ±4 |  |  | KV |

(註1)  Input pin 指SET_PT、KEY、LINE、TEST、DT_CIN、

DV、DI、DL、OSC2、OSC1

IIL表示input pin = 0，IIH表示input pin = VDD

(註2)  Output pin 指RESET、HEAT、WARM、DT_COUT

(註3)  Human body mode , refer to MIL-STD-883D Method 3015.7

KL0173 Application

(A)  Application circuit

當電源頻率不同時，除了檢知溫度的外接電容需調整外，對於其他功能並無太大影響。例如：60Hz時配合TDK104JC使用外加電阻5.1kΩ (1﹪)、電容0.0047uf，50Hz時直接把電容改為0.0047uf ×60 ÷50＝0.00564uf。

分壓用電阻視實際需要而調整。