LED驱动电源技术规格初稿

| 序号 | 技术指标明细 | 注 明 | 评估结果 |
| --- | --- | --- | --- |
| 001 | Output Operating Frequency＞=120Hz(不包括调光) | 必须满足 |  |
| 002 | Transient Protection shall comply with IEEE C.62.41-1991, Class A operation. The line transient shall consist of seven strikes of a 100kHz ring wave, 2.5kV level, for both common mode and differential mode. Refer to IEEE C.62.41-1991 Recommended Practice for Surge Voltages in Low- voltage AC Power Circuits (obsolete) C.62.41.1.2, C.62.41.2 (new standard) | 必须满足 |  |
| 003 | Power supply noise to in Class A sound rating (not exceed 24 dB) | 必须满足 |  |
| 004 | Off-state power – luminaires shall not draw power in the off-state | 必须满足 |  |
| 005 | 输入电压：85-264V（220-240V必须达到，希望达到85-264V）交 流(Transient Protection C.62.41.1.2, C.62.41.2) | 必须满足 |  |
| 006 | 正常工作时的输出电流 ： - 恒流 - 输出电流0.4 A±5% （六串或七串1W） | 必须满足 暂定 |  |
| 007 | 输出过压保护：具有输出过压保护 | 必须满足 |  |
| 008 | 工作温度：-10℃～+65℃ （散热器温度<65C） | 必须满足 |  |
| 009 | 效率：>90% PF>0.9 (ANSI C82.77) | 必须满足 |  |
| 010 | 调光100%-20% （采用市场上兼容的前波段后波段） | 必须满足 |  |
| 012 | EMI (FCC 47 CFR) | 必须满足 |  |
| 013 | 外形尺寸：（见图纸附件一） | 必须满足 |  |

附件一

Input

GU10 85-264V

+（红）

--（黑）

接LED基板

Teflon线

外白色塑料壳