| SPECIFICATION FOR APPROVAL |  |  | CUSTOMER: |  |
| --- | --- | --- | --- | --- |
| LONGDING PART: |  | REV: | CUSTOMER PART NO.: |  |
|  |  | A |  |  |
| PART NAME: |  | SPEC | ISSUE DATE: |  |
| CONTENT： page 1.Configuration and Dimension--------------------------------------4 2.Winding specification---------------------------------------------------5 3.Electrical characteristics------------------------------------------------6 4.Material list-------------------------------------------------------------7 5.Reliability tests---------------------------------------------------------8 |  |  |  |  |
| APPROVAL BY | CHECK BY |  |  | DRAWING BY |

| SPECIFICATION FOR APPROVAL |  | CUSTOMER: |
| --- | --- | --- |
| LONGDING PART: | REV: | CUSTOMER PART NO.: |
|  | A |  |
| PART NAME: | SPEC | ISSUE DATE: |
| 1.Configuration and Dimension UNIT：mm： NOTS：1.GAP 放于PIN1---5侧,磁芯接缝须点胶. 2.成品PIN4 CUT OFF.产品侧面贴标签：ZST 5V/±12V-26V 3.3VAC 3.绕制时一定要用宽3.8mm厚0.2的挡墙,起头收头在顶部.套管要深入挡墙,起收头用固定胶带打直角出线. 4.W2线改为2UEW0.17线,密绕2层. 5.W3改为0.3*2P密绕一层,绕制时骨架反过来插入机台,直接挂PIN5开始绕线.W5,W6和W7,W8为同层串绕.W5,W7靠近PIN1—5 W6,W8靠近PIN6—12. 6.产品必须抽真空. |  |  |

| SPECIFICATION FOR APPROVAL |  | CUSTOMER: |
| --- | --- | --- |
| LONGDING PART: | REV: | CUSTOMER PART NO.: |
|  | A |  |
| PART NAME: | SPEC | ISSUE DATE: |

| SPECIFICATION FOR APPROVAL |  | CUSTOMER: |
| --- | --- | --- |
| LONGDING PART: | REV: | CUSTOMER PART NO.: |
|  | A |  |
| PART NAME: | SPEC | ISSUE DATE: |
| 3.Electrical characteristics ELECTRICAL CHARACTERISTICS. NO; ITEM; TEST TERMINAL; SPECIFICATION; TEST INSTRUMENT. 1; INDUCTANCE @10KHz,1.0V; L(3---5); 1.2mH±10%; CH1062 LCR METER. 2; PRIMARY LEAKAGE INDUCTANCE @10KHz,1.0V; Lk(3---5) PIN1—2 AND SEC SHORT; 250uHMAX; CH1062 LCR METE. 3; DCR; R(3—5); 4.0ΩMAX; CH502B METER. 3-1.HI-POT TEST: ITEM; HI-POT . VOLTAGE. PRI TO SEC; AC 3.5KV/M 50/60Hz 5mA. PRI TO CORE; AC 1.5KV/M 50/60Hz 5mA. SEC TO CORE; AC 1.5KV/M 50/60Hz 5mA. 3-2. INSULATION RESISTANCE: THE INSULATION RESISTANCE BETWEEN COIL TO CORE AND COIL TO COIL IS OVER 100MΩ AT DC500V. |

| SPECIFICATION FOR APPROVAL |  | CUSTOMER: |
| --- | --- | --- |
| LONGDING PART: | REV: | CUSTOMER PART NO.: |
|  | A |  |
| PART NAME: | SPEC | ISSUE DATE: |
| 4.Material list NO; SUB PART; REW MATERIAL. MATERIAL; SUPPLIER OF THE MATERIAL; TEMP; UL NO.. 1; BOBBIN; T375J94V-0(BLACK); SHULIN CO.,LTD; 150℃; E59481(S). 2; CORE; FERRITE CORE EEL16 PC40; KANGSHUN ELECTRIC CO.,LTD XINLI ELECTRIC CO.,LTD; N/A; N/A. 3; WIRE; 2UEW; JU FENG ELECTRIC CO.,LTD; 130℃; E206882. 4; TAPE; #11660,#06660; YA HUA ADHESIVE TAPE CO.,LTD; 130℃; E178516(N). 5; EXPOX; E-500; LIDUO ELECTRIC CO.,LTD; N/A; N/A. 6; TUBE; T.F.L; GREAT HONGDING OR EQU; 200℃; E158258(S). 7; PIN; TINNED COPPER WIRE; OPTIONAL; N/A; N/A. 8; VARNISH; BC-346A (HELLCAL COIL); JOHNC DOLPH CO.,LTD; 200℃; E51047(M). |

| SPECIFICATION FOR APPROVAL |  | CUSTOMER: |
| --- | --- | --- |
| LONGDING PART: | REV: | CUSTOMER PART NO.: |
|  | A |  |
| PART NAME: | SPEC | ISSUE DATE: |
| 5.Reliability tests: NO; TESTITEM; TEST CONDITION; SPECIFICATION. 1; HEAT RESISTANCE TEST; TEMP:85℃±2℃ TIME:96HRS(MIN); *INDUCTANCE:WITHIN±5%OF INITIAL VAIUE *THE ORTHER FUNCTIONS MUST MEET THE SPEC. 2; COLD RESISTANCE TEST; TEMP:25℃±2℃ TIME:96HRS(MIN); *INDUCTANCE:WITHIN±5%OF INITIAL VAIUE *THE ORTHER FUNCTIONS MUST MEET THE SPEC. 3; HUMIDITY TEST; TEMP:40℃±2℃ TEMP:90-95% TIME:500HRS(MIN); *INDUCTANCE:WITHIN±5%OF INITIAL VAIUE *THE ORTHER FUNCTIONS MUST MEET THE SPEC. 4; HIGH TEMP LIFE; TEMP:85℃±2℃ TIME:500HRS(MIN); *INDUCTANCE:WITHIN±5%OF INITIAL VAIUE *THE ORTHER FUNCTIONS MUST MEET THE SPEC. 5; THERMAL CYCLE TEST; STEP1: 25℃±2℃60±10MIN STEP2:AMBIENT TEMP STEP3: 80℃±2℃ STEP4:AMBIENT TEMP STEP1-4,5 CYCLES; *INDUCTANCE:WITHIN±5%OF INITIAL VAIUE *THE ORTHER FUNCTIONS MUST MEET THE SPEC. 6; TERMINAL STRENGTH; TENSILE:1.0KG (MIN)TEMP:30±5SEC; *INDUCTANCE:WITHIN±5%OF INITIAL VAIUE *THE ORTHER FUNCTIONS MUST MEET THE SPEC. 7; SOLDER ABILITY; TEMP:235℃±5℃ TIME:3±0.5SEC; *SOLDER ATTACHMENT95% ABOVE. 8; SOLDERING HEAT RESISTANCE; TEMP:260℃±5℃ TIME:5±0.1SEC; *INDUCTANCE:WITHIN±5%OF INITIAL VAIUE *THE ORTHER FUNCTIONS MUST MEET THE SPEC. |