Appendix A

Electrical Test For 1881-6.0V1.50A

- UUT Information         :

- PSU Model Rev.                      P1

- Input Range                         90 - 264Vac

- Transformer Rev.                    P1

- PCB Rev.                           P1

- Sample No.

- Test Date :	        04-25, 2009

- Tested by :      Alex Du

- Test Equipment:

1．Test Equipment

### CHROMA Programmable AC Source 6120

### INSTEK Power Meter GPM8212

### FLUKE Multimeter 45

### Prodigit  Electronic Load   3302C

### ITECH High voltage Programmable Electronic Load IT8512B

### Agilent DSO6034A  Digital Phosphor Oscilloscope (300MHZ)

### TEKTRONIX TM502A-A6302 current probe passiv 2mA;10mA/mV

2.Electrical Test reports

2.1 Test  Set-up

POWER

ANALYZER

UNIT

UNDER

TEST

AC

SOURCE

ELECTRONIC

LOAD

Fig.1.1 Input  &  Output Test  Set-up

2.2 Load Setup

Unit: A

| Channel |  | +6.0V |
| --- | --- | --- |
| Loading (A) | Max. | 1.50A |
|  | Typ. | 1.00A |
|  | Min. | 0A |

2.3 Output Regulation

1) Requirement              Unit: V

| Channel |  | +6.0V |
| --- | --- | --- |
|  | Max. | 6.30 |
| Requirement | Typ. | 6.00 |
|  | Min. | 5.70 |

2) Testing Data       Unit: V

| Vin(Vac) | Loading | +6.0V |
| --- | --- | --- |
| 90 | Min. | 6.11 |
|  | Typ. | 6.03 |
|  | Max. | 5.95 |
| 120 | Min. | 6.12 |
|  | Typ. | 6.03 |
|  | Max. | 5.94 |
| 200 | Min. | 6.11 |
|  | Typ. | 6.05 |
|  | Max. | 5.93 |
| 240 | Min. | 6.13 |
|  | Typ. | 6.03 |
|  | Max. | 5.95 |
| 264 | Min. | 6.12 |
|  | Typ. | 6.03 |
|  | Max. | 5.93 |
| MAXIMUM |  | 6.12 |
| MINIMUM |  | 5.93 |
| RESULT |  | OK |

2.4 Output Ripple & Noise

- Requirement

Unit: mV

| Channel | +6.0V |
| --- | --- |
| Requirement(mVp-p) | 100mV |

2) Testing Data

Unit: mV

| Vin(Vac) | Loading | +6.0V |
| --- | --- | --- |
| 90 | Max. | 68 |
|  | Typ. | 43 |
|  | Min. | 21 |
| 120 | Max. | 63 |
|  | Typ. | 48 |
|  | Min. | 22 |
| 200 | Max. | 60 |
|  | Typ. | 56 |
|  | Min. | 26 |
| 240 | Max. | 59 |
|  | Typ. | 37 |
|  | Min. | 27 |
| 264 | Max. | 70 |
|  | Typ. | 56 |
|  | Min. | 35 |
| MAXIMUM |  | 70 |
| RESULT |  | OK |

2.5 OCP test

1) Requirement

Unit: A

| Channel | +6.0V |
| --- | --- |
| Requirement (MIN) | 1.6 |

2) Testing Data

Unit: A

| Vin(Vac) | +6.0V |
| --- | --- |
| 90 | 1.65 |
| 120 | 1.78 |
| 200 | 2.10 |
| 240 | 2.31 |
| 264 | 2.55 |
| MAXIMUM | 2.55 |
| MINIMUM | 1.65 |
| RESULT | OK |

2.6 Short Test Report

Testing Data

| Vin(Vac) | Loading | +6.0V |
| --- | --- | --- |
| 90 | Max. | OK |
|  | Typ. | OK |
|  | Min. | OK |
|  | Min. | OK |
| 120 | Max. | OK |
|  | Typ. | OK |
|  | Min. | OK |
| 200 | Max. | OK |
|  | Typ. | OK |
|  | Min. | OK |
| 240 | Max. | OK |
|  | Typ. | OK |
|  | Min. | OK |
| 264 | Max. | OK |
|  | Typ. | OK |
|  | Min. | OK |
| RESULT |  | OK |

TEST RESULT : OK

2.7  Heavy Load Test

Put the units into the worst condition for 24hrs, measure the output

And current at both the beginning and end.

| The unit may have functional degrading under the testing, however, Input | 23C/110Vac/50Hz |
| --- | --- |
| Load | 2.0A |
| Vout (V) at the beginning | 6.14V |
| Iout (mA) at the beginning | 1500mA |
| Duration | 24hrs |
| Vout(V) at the end | 6.12V |
| Iout(mA) at the end | 1500mA |

when going back to normal condition, any permanent functional loss is unacceptable.

2.8 HI-pot  Test:

TEST Condition:

Primary to Secondary:

●Test voltage: 3.75KVac

●Test time: 60S

●Cut off current: 10mA

●Room temperature: 22℃

●Humidity: 80%

●Atmospheric pressure: 100Kpa

●Insulations resistance (>100MΩ)

TEST RESULT:  OK