iP7302 7W LED Driver with iP7701 Test Report

7W LED Driver Demo Board Base on

PFC Flyback Switch iP7302

Part Number: IS7302_7701_DB02 (18V~21V/0.3A)

Outline Dimension: L x W x H (48.8mm / 31.0mm / 20.0mm)

Photo1. Top View

Photo2. Side View

Photo3. Bottom View

Contents

1. Test Equipments ---------------------------------------------------------------------------------------------------------- 4

2. Demo Board Specification --------------------------------------------------------------------------------------------- 4

3. Demo Board Test Item List -------------------------------------------------------------------------------------------- 5

4. Electronic Characteristics Test Data ------------------------------------------------------------------------------- 6

4.1 Efficiency -----------------------------------------------------------------------------------------------------------------6

4.2 Ripple & Noise ----------------------------------------------------------------------------------------------------------7

4.3 Over Shoot  -----------------------------------------------------------------------------------------------------------10

4.4 Time Sequence Characteristic ----------------------------------------------------------------------------------12

4.5 Voltage stress on MOSFET----------------------------------------------------------------------------------------15

4.6 Current Limit ---------------------------------------------------------------------------------------------------------17

5. Schematic -------------------------------------------------------------------------------------------------------------------18

6. BOM List ---------------------------------------------------------------------------------------------------------------------19

7. PCB Layout -----------------------------------------------------------------------------------------------------------------20

7.1 Top silkscreen --------------------------------------------------------------------------------------------------------20

7.2 Bottom Silkscreen ---------------------------------------------------------------------------------------------------20

7.3 Bottom layer -----------------------------------------------------------------------------------------------------------20

8. Transformer-----------------------------------------------------------------------------------------------------------------21

8.1 Schematic & Winding -----------------------------------------------------------------------------------------------21

9. EMI Test ----------------------------------------------------------------------------------------------------------------------22

9.1 Conduction (EN55015 Class B) ---------------------------------------------------------------------------------22

10. Revision History ---------------------------------------------------------------------------------------------------------30

Figure List

Figure 1 The waveform of Ripple & Noise at Vin = 90Vac/60Hz & full-load --------------------------------8

Figure 2 The waveform of Ripple & Noise at Vin = 110Vac/60Hz & full-load ------------------------------8

Figure 3 The waveform of Ripple & Noise at Vin = 220Vac/50Hz & full-load -----------------------------9

Figure 4 The waveform of Ripple & Noise at Vin = 264Vac/50Hz & full-load -----------------------------9

Figure 5 The waveform of Over shoot at Vin = 90Vac/60Hz & full-load ------------------------------------10

Figure 6 The waveform of Over shoot at Vin = 110Vac/60Hz & full-load -----------------------------------11

Figure 7 The waveform of Over shoot at Vin = 220Vac/50Hz & full-load ------------------------------------11

Figure 8 The waveform of Over shoot at Vin = 264Vac/50Hz & full-load -----------------------------------12

Figure 9 The waveform of Turn on Time at Vin = 90Vac/60Hz & full-load -------------------------------- 13

Figure 10 The waveform of Turn on Time at Vin = 110Vac/60Hz & full-load -----------------------------13 Figure 11 The waveform of Turn on Time at Vin = 220Vac/50Hz & full-load -------------------------------- 14

Figure 12 The waveform of Turn on Time at Vin = 264Vac/50Hz & full-load -----------------------------14

Figure 13 The waveform of Drain at Vin = 264Vac/50Hz & full-load -----------------------------------------15

Figure 14 The waveform of Drain at Vin = 264Vac/50Hz & full-load startup-------------------------------- 16

Figure 15 The waveform of rectifier at Vin = 264Vac/50Hz & full-load -------------------------------------16

Figure 16 The waveform of rectifier at Vin = 264Vac/50Hz & full-load startup------------------------------17

Figure 17 The waveform of Conduction-L at Vin = 110Vac/60Hz & full-load ---------------------------- 22

Figure 18 The waveform of Conduction-L at Vin = 110Vac/60Hz & full-load ---------------------------- 24

Figure 19 The waveform of Conduction-L at Vin = 110Vac/60Hz & full-load ---------------------------- 26

Figure 20 The waveform of Conduction-L at Vin = 110Vac/60Hz & full-load ---------------------------- 28

1. Test Equipments

| Name | Mark |
| --- | --- |
| AC Source | Extech 6905 |
| Oscilloscope | Tektronix DPO3014 |
| Power Meter | Voltech PM100 |
| Load | 7W LED |
| True RMS Multi-meter | Fluke 287 |
| Differential Probe | LDP-6002 |

2. Demo Board Specification

| Parameter | Specification |
| --- | --- |
| Input Voltage | 90Vac~264Vac |
| Input Frequency | 47Hz~63Hz |
| Output Voltage and Current | 20V/0.3A |
| Output Power | 6.3W (max.) |
| Efficiency | > 80% Full Load |
| Conduction | EN55015 Class B |

3. Demo Board Test Item List

All test conditions is base on ambient temperature 25℃;

| Test item | Specification | Result |
| --- | --- | --- |
| Efficiency | >80% Full Load | Pass |
| Ripple & Noise | --- | --- |
| Overshoot | --- | --- |
| Turn on time | <2S | Pass |
| Voltage stress on MOSFET | ≤650V | Pass |
| Constant Current | 0.3A (±3%) | Pass |
| Short Circuit Protection | Recovery | Pass |
| EMI Test - Conduction | EN55015 Class B | Pass |

4. Electronic Characteristics Test Items List

All test conditions is base on ambient temperature 25℃;

4.1 Efficiency

Test Condition:

| We chose input voltage value, including 90V, 100V, 110V, 220V, 230V, 264Vand 100% Load Current, Then Calculate the efficiency. | 90Vac | 100Vac | 110Vac |
| --- | --- | --- | --- |
| Vout (V) | 19.91 | 19.9 | 19.91 |
| Iout (mA) | 297.8 | 298 | 297.9 |
| Pin (W) | 6.97 | 6.94 | 6.92 |
| Pout (W) | 5.93 | 5.93 | 5.93 |
| PF | 0.984 | 0.988 | 0.989 |
| η (%) | 85.07 | 85.45 | 85.7 |

|  | 220Vac | 230Vac | 264Vac |
| --- | --- | --- | --- |
| Vout (V) | 19.9 | 19.9 | 19.9 |
| Iout (mA) | 298.3 | 298.2 | 298.3 |
| Pin (W) | 7.08 | 7.11 | 7.23 |
| Pout (W) | 5.94 | 5.93 | 5.94 |
| PF | 0.966 | 0.958 | 0.916 |
| η (%) | 83.84 | 83.46 | 82.1 |

4.2 Ripple & Noise

Test Condition:

The ripple & noise are measured by using 20MHz bandwidth Limited oscilloscope with a 10uF low impedance electronic capacitor and a 0.1uF Ceramic capacitor.

Test data and results are as follows:

| Vin | V(p-p)Full Load | Spec | Result | Note |
| --- | --- | --- | --- | --- |
| 90V/60Hz | 4.6 V | -- | -- | Figure 1 |
| 110V/60Hz | 4.6 V | -- | -- | Figure 2 |
| 220V/50Hz | 5.0 V | -- | -- | Figure 3 |
| 264V/50Hz | 4.8 V | -- | -- | Figure 4 |

Full Load: 20V/0.3A

Figure 1  The waveform of Ripple & Noise at Vin = 90Vac/60Hz & full-load

Figure 2  The waveform of Ripple & Noise at Vin = 110Vac/60Hz & full-load

Figure 3  The waveform of Ripple & Noise at Vin = 220Vac/50Hz & full-load

Figure 4  The waveform of Ripple & Noise at Vin = 264Vac/50Hz & full-load

4.3  Over Shoot

Test Condition:

Over Shoot value test point are all at PCB end

Test data and results are as follows:

| Vin | Load | Test Item | Spec(%) | Test Data | Result | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 90V/60Hz | Full Load | Over Shoot | -- | 15% | -- | Figure5 |
| 110V/60Hz | Full Load | Over Shoot | -- | 20% | -- | Figure6 |
| 220V/50Hz | Full Load | Over Shoot | -- | 36% | -- | Figure7 |
| 264V/50Hz | Full Load | Over Shoot | -- | 42% | -- | Figure8 |

Figure 5  The waveform of Over shoot at Vin = 90Vac/60Hz & Full load

Figure 6  The waveform of Over Shoot at Vin = 110Vac/60Hz & Full load

Figure 7  The waveform of Over Shoot at Vin = 220Vac/50Hz & Full load

Figure 8  The waveform of Over Shoot at Vin = 264Vac/50Hz & Full load

4.4 Time Sequence Characteristic

Test Condition:

Turn on delay time , Hold up time, Rising time and Falling time are all base on full load.

Test data and results are as follows:

| Item | Vin(ac) | Test Data | Spec | Result | Note |
| --- | --- | --- | --- | --- | --- |
| Turn on delay time | 90V/60Hz | 1.77S | <2S | Pass | Figure 9 |
|  | 110V/60Hz | 1.43S | <2S | Pass | Figure 10 |
|  | 220V/50Hz | 685mS | <2S | Pass | Figure 11 |
|  | 264V/50Hz | 568mS | <2S | Pass | Figure 12 |

Figure 9   The waveform of Turn on Time at Vin = 90Vac/60Hz & full-load

Figure 10   The waveform of Turn on Time at Vin = 110Vac/60Hz & full-load

Figure 11   The waveform of Turn on Time at Vin = 220Vac/50Hz & full-load

Figure 12   The waveform of Turn on Time at Vin = 264Vac/50Hz & full-load

4.5 Voltage stress on MOSFET

Test Condition:

Measure the voltage on MOSFET and secondary rectifiers on full load.

| Vin(V) | State | Stress on MOSFET | Note | Rating | Stress on rectifier | Note | Rating |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 264 | Normal | 596V | Figure13 | 650V | 86.4V | Figure15 | 100V |
|  | Startup | 612V | Figure14 |  | 92.8V | Figure16 |  |

Figure 13  The waveform of Drain at Vin = 264Vac/50Hz & full-load

Figure 14  The waveform of Drain at Vin = 264Vac/50Hz & full-load Startup

Figure 15  The waveform of rectifier at Vin = 264Vac/50Hz & full-load

Figure 16  The waveform of rectifier at Vin = 264Vac/50Hz & full-load Startup

4.6 Constant Current

| Input Voltage | Current Limit Value(A) | Result |
| --- | --- | --- |
| 90V /60Hz | 0.298 | PASS |
| 100V /60Hz | 0.298 |  |
| 110V /60Hz | 0.298 |  |
| 220V /50Hz | 0.298 |  |
| 230V /50Hz | 0.298 |  |
| 264V /50Hz | 0.298 |  |

5. Schematic

6. BOM List

| 序號 | 品 名 | 規 格 | 數量 | 製程 | 打件位置 |
| --- | --- | --- | --- | --- | --- |
| 10 | 晶片電阻 | RES SMD 1/4W 820K F 1206 | 2 | SMD | R1,R2 |
| 20 | 晶片電阻 | RES SMD 1/4W 330K F 1206 | 2 | SMD | R3,R4 |
| 30 | 晶片電阻 | RES SMD 1/4W 2R7 F 1206 | 1 | SMD | R7 |
| 40 | 晶片電阻 | RES SMD 1/8W 20K F 0805 | 2 | SMD | R5,R9 |
| 50 | 晶片電阻 | RES SMD 1/8W 0R F 0805 | 1 | SMD | R6 |
| 60 | 晶片電阻 | RES SMD 1/8W 68K F 0805 | 1 | SMD | R8 |
| 70 | 晶片電阻 | RES SMD 1/8W 180K F 0805 | 1 | SMD | R10 |
| 80 | 晶片電阻 | RES SMD 1/8W 10K F 0805 | 3 | SMD | R11,R13,R14 |
| 90 | 晶片電阻 | RES SMD 1/8W 47K F 0805 | 2 | SMD | R12,R15 |
| 100 | 晶片電阻 | RES SMD 1W 0R33 F 2512 | 1 | SMD | R16 |
| 110 | 晶片電阻 | RES SMD 1/8W 1K F 0805 | 1 | SMD | R17 |
| 120 | 晶片電容 | CAP SMD NPO 333 25V 0805 | 1 | SMD | C3 |
| 130 | 晶片電容 | CAP SMD NPO 104 25V 0805 | 4 | SMD | C4,C5,C6,C7 |
| 140 | PWM IC | iP7302 SO8 | 1 | SMD | IC1 |
| 150 | POWER MOS | iH0265T SO8 | 1 | SMD | IC2 |
| 160 | REFERENCE IC | iP7701 SOT-23-6L | 1 | SMD | IC3 |
| 170 | 橋式整流器 | B6S 600V/0.8A | 1 | SMD | BD1 |
| 180 | BIPOLAR | 2N2222 | 1 | SMD | Q2 |
| 190 | 二極體 | 20V ZENER | 1 | SMD | Z2 |
| 200 | 二極體 | 12V ZENER | 1 | SMD | Z3 |
| 210 | 二極體 | 800V/1A UF4007 ULTRAFAST | 1 | DIP | D1 |
| 220 | 二極體 | 1N4004 400V/1A GENERAL | 1 | DIP | D2 |
| 230 | 二極體 | 100V/2A SR2100 SCHOTTKY | 1 | DIP | D3 |
| 240 | 二極體 | P6KE200A TVS | 1 | DIP | Z1 |
| 250 | X-CAP | 0.15uF/300VAC | 1 | DIP | X1 |
| 260 | Y-CAP | 332M | 1 | DIP | Y1 |
| 270 | 金屬皮膜電容 | 103/630V | 1 | DIP | C1 |
| 280 | 金屬皮膜電容 | 473/400V | 1 | DIP | C2 |
| 290 | 鋁質電解電容 | 22uF/25V LONG LIFE | 1 | DIP | E1 |
| 300 | 鋁質電解電容 | 220uF/35V LONG LIFE | 1 | DIP | E2 |
| 310 | 光耦合器 | PC817 DIP4 | 1 | DIP | Q1 |
| 320 | 電感 | 1mH | 2 | DIP | L1,L2 |
| 330 | 變壓器 | EFD15 | 1 | DIP | T1 |

7. PCB Layout

7.1 Top Silkscreen

7.2 Bottom Silkscreen

7.3 Bottom Layer

8. Transformer

8.1 Schematic & Winding

9. EMI Test

9.1 Conduction (EN55015 Class B)

Figure 17   The waveform of Conduction – L at Vin =110Vac/60Hz & full-load

Figure 18   The waveform of Conduction – N at Vin =110Vac/60Hz & full-load

Figure 19   The waveform of Conduction – L at Vin =220Vac/50Hz & full-load

Figure 20   The waveform of Conduction – N at Vin =220Vac/50Hz & full-load

10.  Revision History

| Ver. | Date | Change Notice |
| --- | --- | --- |
| 1.0 | 2010/05/04 | Original |
| 1.1 | 2010/05/18 | IC3 change to iP7701 R16 change to R330/2512 |